HealthSync Indonesia

IoT & Wearable Integration Specification

Protokol Data Smartwatch, MQTT & Real-time Monitoring

Versi 1.0 | 14 September 2026

Kategori: Arsitektur Teknis

# 1. Supported Devices (MVP)

| Device | Protocol | Sensors | Battery (Continuous) | SDK |
| --- | --- | --- | --- | --- |
| Samsung Galaxy Watch 6 | BLE 5.0 + WiFi | HR, SpO2, Temp, ECG, Accel | ~24 jam | Samsung Health SDK |
| Apple Watch Series 9 (Phase 2) | BLE 5.0 | HR, SpO2, Temp, ECG, Accel | ~18 jam | HealthKit |

# 2. Data Collection Specification

## 2.1 Sampling Rates

| Metric | Normal Mode | Alert Mode | Unit | Range Valid |
| --- | --- | --- | --- | --- |
| Heart Rate | 1x / 5 detik | 1x / 1 detik | BPM | 30-250 |
| SpO2 | 1x / 60 detik | 1x / 5 detik | % | 50-100 |
| Blood Pressure | 1x / 30 menit | N/A | mmHg | 60-250 / 40-150 |
| Skin Temperature | 1x / 5 menit | 1x / 30 detik | °C | 30-45 |
| Activity/Steps | 1x / 10 detik | N/A | steps | 0-100000 |
| Battery Level | 1x / 5 menit | 1x / 1 menit | % | 0-100 |

## 2.2 MQTT Topic Structure

healthsync/devices/{device_id}/vitals — Real-time vital signs (QoS 1)

healthsync/devices/{device_id}/status — Device online/offline/battery (QoS 0)

healthsync/devices/{device_id}/config — Downlink: update sampling rate (QoS 1)

healthsync/patients/{patient_id}/alerts — Alert notifications (QoS 2)

healthsync/patients/{patient_id}/commands — Emergency commands (QoS 2)

healthsync/system/heartbeat — Broker health check (QoS 0)

## 2.3 Data Packet Format (JSON)

{
  "device_id": "GW6-XXXX-XXXX",
  "patient_id": "uuid-here",
  "timestamp": "2025-01-15T08:30:00Z",
  "vitals": {
    "heart_rate": 72,
    "spo2": 98,
    "systolic_bp": 120,
    "diastolic_bp": 80,
    "temperature": 36.5,
    "activity_level": "resting"
  },
  "battery_level": 85,
  "signal_strength": -45,
  "firmware_version": "2.1.3"
}

# 3. Alert Thresholds

| Metric | Warning | Critical | Escalation Action |
| --- | --- | --- | --- |
| Heart Rate | >120 atau <50 BPM | >150 atau <40 BPM | Warning: notify. Critical: escalate |
| SpO2 | <92% | <88% | Warning: notify. Critical: escalate |
| Temperature | >38.5°C | >39.5°C | Warning: notify. Critical: escalate |
| Systolic BP | >160 mmHg | >180 mmHg | Warning: notify. Critical: escalate |
| Diastolic BP | >100 mmHg | >120 mmHg | Warning: notify. Critical: escalate |

# 4. Escalation Ladder

| Level | Trigger | Action | Timeout |
| --- | --- | --- | --- |
| Level 1 | Threshold Warning | Push notif ke pasien: 'Apakah Anda OK?' | 60 detik |
| Level 2 | 60 detik tanpa respons ATAU Threshold Critical | Alert ke Command Center RS | 2 menit |
| Level 3 | 2 menit tanpa respons dari Level 2 | Auto-dispatch ambulans + SMS keluarga | - |

# 5. Reliability & QoS

| Parameter | Value | Rationale |
| --- | --- | --- |
| MQTT QoS (vitals) | 1 (At least once) | Data tidak boleh hilang |
| MQTT QoS (alerts) | 2 (Exactly once) | Alert tidak boleh duplikat/hilang |
| Keep-alive interval | 30 detik | Deteksi device offline cepat |
| Retry policy | 3x exponential backoff | Handle network instability |
| Local buffer capacity | 1000 readings | Store & forward saat offline |
| TLS version | 1.3 (mandatory) | Security |
| Data compression | gzip untuk batch > 10 readings | Bandwidth efficiency |

# 6. Pairing Flow

1. Pasien buka app → menu 'Perangkat Saya' → 'Tambah Perangkat'

2. App scan BLE devices dalam radius 10 meter

3. Pasien pilih device dari list → konfirmasi pairing code (6 digit)

4. App kirim registration ke backend (device_id ↔ patient_id mapping)

5. Backend generate device certificate untuk mTLS

6. Test data transmission: 5 detik sample → validasi

7. Konfirmasi berhasil → monitoring aktif → notifikasi ke Command Center RS

# 7. Failure Handling

| Scenario | Detection | Handling | Recovery |
| --- | --- | --- | --- |
| BLE disconnected | Phone app timeout 10s | Retry 3x → notify pasien | Auto-reconnect saat dalam range |
| Phone offline | MQTT connection lost | Smartwatch buffer data | Sync saat reconnect (max 1000 readings) |
| Cloud unreachable | MQTT publish fail | Phone buffer + retry backoff | Batch upload saat available |
| Battery < 10% | Battery reading | Alert ke pasien + Command Center | Suggest charging |
| Sensor malfunction | Out-of-range values 3x | Flag data unreliable | Notify pasien, skip alerts |
| Firmware outdated | Version check on connect | Prompt update | Block jika < minimum version |